RX

SPECIFICATIONS

| August 2022. Tentative data, pending homologation |  |  | RX 350 | RX 350h | RX 450h+ | RX 500h |
| --- | --- | --- | --- | --- | --- | --- |
| DIMENSIONS & WEIGHT |  |  |  |  |  |  |
| Overall | Length | mm | 4890 |  |  |  |
|  | Width | mm | 1920 |  |  |  |
|  | Height | mm | 1695 |  |  |  |
| Wheelbase |  | mm | 2850 |  |  |  |
| Track | Front | mm | 1655 |  |  |  |
|  | Rear | mm | 1695 |  |  |  |
| Overhang | Front | mm | 1080 |  |  |  |
|  | Rear | mm | 960 |  |  |  |
| Hip room | Front | mm | 1440 |  |  |  |
|  | Rear | mm | 1420 |  |  |  |
| Couple distance | Front to rear | Mm | 1012 |  |  |  |
| Min. Running Ground Clearance |  | mm | 185 | 183 | 182 | 183 |
| Angle of Approach | Empty | degrees | 15 | 15 | 15 | 15 |
| Angle of Departure | Empty | degrees | 24 | 24 | 24 | 23 |
| Interior | Length | mm | 2053 |  |  |  |
|  | Width | mm | 1522 |  |  |  |
|  | Height | mm | 1204 |  |  |  |
| Seating capacity |  | pax | 5 |  |  |  |
| Cargo Space | VDA (fully loaded) | litre | 612 |  |  |  |
|  | VDA (under tonneau cover) | litre | 461 |  |  |  |
| Gross Vehicle Weight | Total | kg | 2590 | 2660 | 2780 | 2750 |
|  | Front | kg | 1255 | 1235 | 1330 | 1285 |
|  | Rear | kg | 1335 | 1425 | 1450 | 1465 |
| Curb Mass | Total | kg | 1920 to 2000 | 1965 to 2065 | 2110 to 2240 | 2100 to 2190 |
|  | Front | kg | 1120 to 1160 | 1105 to 1135 | 1200 to 1230 | 1175 to 1185 |
|  | Rear | kg | 800 to 840 | 860 to 960 | 910 to 1010 | 925 to 1005 |
| Towing Capacity | With Brake | kg | 2000 |  |  |  |
|  | Without Brake | kg | 750 |  |  |  |
| FUEL |  |  |  |  |  |  |
| Fuel Type |  |  | Petrol |  |  |  |
| Recommended Octane Number |  |  | 95 or more (octane) |  |  |  |
| Fuel Tank Capacity |  | litre | 67.5 | 65 | 55 | 65 |

|  |  |  |  | RX 350 | RX 350h | RX 450h+ | RX 500h |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENGINE |  |  |  |  |  |  |  |  |
| No. of Cyls. & Arrangement |  |  |  | 4-cylinder, In-line |  |  |  |  |
| Displacement |  | litre |  | 2.4 | 2.5 | 2.5 | 2.4 |  |
| Type |  |  |  | 4-cylinder, In-line turbocharged | 4-cylinder, In-line | 4-cylinder, In-line | 4-cylinder, In-line turbocharged |  |
| Valve Mechanism |  |  |  | 16-valve DOHC, Chain Drive (with Dual VVT-i) | 16-valve DOHC, Chain Drive (With VVT-iE and VVT-i) | 16-valve DOHC, Chain Drive (With VVT-iE and VVT-i) | 16-valve DOHC, Chain Drive (with Dual VVT-i) |  |
| Bore x Stroke |  | mm |  | 87.5 x 99.5 | 87.5 x 103.4 | 87.5 x 103.4 | 87.5 x 99.5 |  |
| Displacement |  | cm2 |  | 2393 | 2487 | 2487 | 2393 |  |
| Compression Ratio |  |  |  | 11.0 : 1 | 14.0 : 1 | 14.0 : 1 | 11.0 : 1 |  |
| Maximum engine output |  | kW (PS) |  | 183 (249) | 140 (190) | 136 (185) | 200 (272) |  |
| Maximum engine torque |  | Nm / rpm |  | 430 / 1,700-3,600 | 239 / 4,300 - 4,500 | 227 / 3,600 - 3,700 | 460 / 2,000 - 3,000 |  |
| MOTOR GENERATOR |  |  |  |  |  |  |  |  |
| Type | front, rear |  |  | - | Permanent Magnet Synchronous Motor |  |  |  |
| Maximum output | front | kW (PS) |  | - | 134 (182) | 134 (182) | 64 (87) |  |
|  | rear | kW (PS) |  | - | 40 (54) | 40 (54) | 76 (103) |  |
| Maximum torque | front | Nm |  | - | 270 | 270 | 292 |  |
|  | rear | Nm |  | - | 121 | 121 | 168.5 |  |
| HYBRID BATTERY |  |  |  |  |  |  |  |  |
| Main battery type |  |  |  | - | Nickel Metal Hydride | Lithium-ion |  | Nickel Metal Hydride |
| Battery Capacity (3HR) |  |  | Ahr | - | 5.0 | 51 |  | 5.0 |
| Number of battery cells |  |  |  | - | 216 | 96 |  | 240 |
| Overall voltage |  |  | kWh | - | n.a. | 18.1 |  | n.a. |
| Maximum system output |  | kW (PS) |  | - | 184 (250) | 227 (309) | 273 (371) |  |
| Max. system torque* |  | Nm |  | n.a. | n.a. | n.a. | 550.8 |  |
| Fuel Consumption** | WLTP combined (high) | l/100km |  | 9.6 | 6.6 | 1.2 | 8.3 |  |
|  | WLTP combined (low) | l/100km |  |  | 6.3 | 1.1 | 8.0 |  |
| CO2 Emissions** | WLTP combined (high) | g/km |  | 219 | 149 | 26 | 189 |  |
|  | WLTP combined (low) | g/km |  |  | 142 | 24 | 182 |  |
| *For the RX 500h, system torque has been calculated using values comparable to maximum torque values for an internal combustion engine of similar output (in-house measurements). The torque characteristics of the RX 500h are equivalent to a petrol-engine-only vehicle that has a maximum of 550.8 Nm of torque. ** Target, pending homologation. For RX 350, target data for combined NEDC cycle is 8.7 l/100km and 197g CO2/km. |  |  |  |  |  |  |  |  |
| PERFORMANCE |  |  |  |  |  |  |  |  |
| Max. Speed |  |  | km/h | 200 | 200 | 200 |  | 210 |
| Acceleration | 0 - 100 km/h |  | sec | 7.6 | 7.9 | 6.5 |  | 6.2 |
| Min. Turning Radius | Tyre |  | m | 5.9 | 5.9 | 5.9 |  | 5.5 |
|  | Body |  | m | 6.3 | 6.3 | 6.3 |  | 5.9 |

|  |  |  |  |  |  |  | RX 350 |  | RX 350h |  | RX 450h+ |  | RX 500h |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRIVETRAIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Drivetrain type |  |  |  |  |  |  | AWD |  | E-four |  | E-Four |  |  | DIRECT4 |  |
| Transaxle / transmission |  | Type |  |  |  |  | 8DAT |  | Hybrid transaxle |  | Hybrid transaxle |  |  | 6DAT |  |
|  |  | Gear ratios |  |  | 1st |  | 5.519 |  | - |  | - |  |  | 4.475 |  |
|  |  |  |  |  | 2nd |  | 3.184 |  | - |  | - |  |  | 2.517 |  |
|  |  |  |  |  | 3rd |  | 2.050 |  | - |  | - |  |  | 1.561 |  |
|  |  |  |  |  | 4th |  | 1.491 |  | - |  | - |  |  | 1.143 |  |
|  |  |  |  |  | 5th |  | 1.234 |  | - |  | - |  |  | 0.851 |  |
|  |  |  |  |  | 6th |  | 1.000 |  | - |  | - |  |  | 0.672 |  |
|  |  |  |  |  | 7th |  | 0.800 |  | - |  | - |  |  | - |  |
|  |  |  |  |  | 8th |  | 0.673 |  | - |  | - |  |  | - |  |
|  |  |  |  |  | Reverse |  | 4.220 |  | - |  | - |  |  | 3.196 |  |
|  |  | Final Drive Gear Ratio |  |  |  |  | 3.329 |  | 3.638 |  | 3.638 |  |  | 3.737 |  |
| Differential Gear Ratio |  | Front / rear |  |  |  |  | 3.329 / 2.277 |  | 3.638 / 10.781 |  | 3.638 / 10.781 |  |  | 3.737 / 10.755 |  |
| Traction Motor |  | Motor Reduction Ratio |  |  |  |  | - |  | 10.781 |  | 10.781 |  |  | 10.755 |  |
| SUSPENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suspension Type |  | Front |  |  |  |  | MacPherson Strut |  |  |  |  |  |  |  |  |
|  |  | Rear |  |  |  |  | Multi Link |  |  |  |  |  |  |  |  |
| Stabilizer Bar |  | Front |  |  |  |  | TORSION BAR |  |  |  |  |  |  |  |  |
|  |  | Rear |  |  |  |  | TORSION BAR |  |  |  |  |  |  |  |  |
| STEERING SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steering Gear Type |  |  |  |  |  |  | Rack and Pinion |  |  |  |  |  |  |  |  |
| Power Steering Type |  |  |  |  |  |  | EPS |  |  |  |  |  |  |  |  |
| Steering Overall Gear Ratio |  |  |  |  |  |  | 14.4 |  |  |  |  |  |  |  |  |
| Lock to Lock |  |  |  |  |  |  | 2.76 |  |  |  |  |  |  |  |  |
| BRAKE SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Brake Type |  | Front & rear |  |  |  |  | Ventilated Disc |  |  |  |  |  |  |  |  |
| Parking Brake Type and Location |  |  |  |  |  |  | EPB_Centre Console Switch |  |  |  |  |  |  |  |  |
| Brake Size |  | Front |  |  |  |  | 18" DISC 28V MID MU, 20" OPPOSED 28V MID MU |  | 18" DISC 28V MID MU, 18" DISC 38V HI MU |  | 18" DISC 38V HI MU |  |  | 20" OPPOSED 28V MID MU, 20" OPPOSED 28V HI MU |  |
|  |  | Rear |  |  |  |  | 18" DISC 18V MID MU |  | 18" DISC 18V MID MU |  | 18" DISC 18V MID MU |  |  | 18" DISC 18V MID MU |  |

# # #